AI-ASSISTED SDN
TRAFFIC ROUTING

Laptop Setup and End-to-End Run Guide

Purpose: Use this guide to install the project on a Windows laptop with WSL2, train the Random Forest model, run the Mininet topology, test all three controllers, and find the generated results.

Tested environment

Windows + WSL2 Ubuntu 22.04 | Mininet 2.3.0 | Open vSwitch 2.17.9 | Python 3.10

Project folder

C:\Users\ROHAN A S GOWDA\OneDrive\Dokumen\computer_network_rohan

# 1. Before You Start

Recommended path: Run the full simulation inside WSL2. Use Windows PowerShell only to install or open WSL; run Mininet and the controllers in Ubuntu terminals.

## What you need

Windows 10 version 2004 or newer, or Windows 11.

Administrator access for the initial WSL installation.

Internet access for Ubuntu packages and Python dependencies.

At least 4 GB free disk space and 8 GB RAM recommended.

The complete project folder on this laptop.

## How the run is organized

| Terminal | Runs as | Purpose |
| --- | --- | --- |
| Ubuntu Terminal 1 | Normal WSL user | Starts Ryu, POX, or the raw OpenFlow controller. |
| Ubuntu Terminal 2 | sudo for Mininet | Starts the six-switch topology and traffic workload. |
| Windows PowerShell | Normal/Admin | Installs WSL and opens additional Ubuntu sessions. |

Important: Only one controller can listen on TCP port 6633 at a time. Stop the current controller with Ctrl+C before starting the next one.

# 2. Install and Verify WSL2

Open PowerShell as Administrator.

Install Ubuntu 22.04 with WSL2.

wsl --install -d Ubuntu-22.04

Restart Windows if requested, then open Ubuntu and create your Linux username and password.

Verify that Ubuntu is using WSL version 2.

wsl -l -v

Expected: The output should list Ubuntu-22.04 with VERSION 2. If it shows version 1, run: wsl --set-version Ubuntu-22.04 2

# 3. One-Time Ubuntu Setup

Open Ubuntu 22.04.

Update package information and install Mininet, Open vSwitch, iperf, Git, and Python tools.

sudo apt update
sudo apt install -y mininet openvswitch-switch iperf python3-pip git

Start Open vSwitch and check that it responds.

sudo service openvswitch-switch start
sudo ovs-vsctl show

Move to the project folder from inside WSL.

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
pwd

Why the quotes matter: The Windows username contains spaces. Keep the double quotes around the WSL project path.

Install the Python packages for the AI, plots, and Ryu controller.

python3 -m pip install --user -r requirements-mininet.txt
export PATH="$HOME/.local/bin:$PATH"

Verify the main tools.

python3 --version
mn --version
ovs-vsctl --version | head -n 1
$HOME/.local/bin/ryu-manager --version

## Optional: make the PATH setting permanent

echo 'export PATH="$HOME/.local/bin:$PATH"' >> ~/.bashrc
source ~/.bashrc

# 4. Train the AI Model

Confirm that you are in the project root.

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"

Generate training data, train the Random Forest, and save the report.

python3 -m ai.train_model

Expected files: models/traffic_rf.joblib, data/training_flows.csv, and results/model_report.json should exist after training.

# 5. Run a Quick Test Without Mininet

This test checks the AI classifier, weighted routing, congestion logic, controller comparison, tables, and graphs. It is the fastest way to confirm the Python side is healthy.

python3 -m experiments.inspect_path_choices
python3 -m experiments.run_comparison --events 240 --output-dir results

## Expected comparison outputs

| File | What it contains |
| --- | --- |
| comparison_summary.csv | Controller-level throughput, delay, install time, CPU, reroutes, and congestion score. |
| comparison_raw.csv | Per-flow measurements used to calculate the summary. |
| throughput_mbps.png | Throughput comparison graph. |
| delay_ms.png | Average delay comparison graph. |
| install_ms.png | Routing-rule installation timing graph. |
| cpu_percent.png | Controller CPU comparison graph. |

Checkpoint: If this step succeeds, the Python dependencies and model are working. Continue to Mininet for the real virtual-network test.

# 6. Full Mininet Run: Ryu

## Terminal 1 - start Ryu

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
export PYTHONPATH="$PWD"
$HOME/.local/bin/ryu-manager --ofp-tcp-listen-port 6633 controllers/ryu_ai_controller.py

Leave it running: Do not close Terminal 1. Open a second Ubuntu terminal for Mininet.

## Terminal 2 - run topology and traffic

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
sudo env PYTHONPATH="$PWD" python3 -m topologies.run_mininet_experiment \
  --controller-ip 127.0.0.1 \
  --controller-port 6633 \
  --profile mixed \
  --duration 30 \
  --output results/mininet_ryu_run.json

Success signal: Mininet should report 0% ping loss and write results/mininet_ryu_run.json. The controller writes decisions to results/ryu_decisions.jsonl.

# 7. Full Mininet Run: POX

POX is installed as a separate source checkout because it is not part of the Python requirements file.

## One-time POX setup

git clone --depth 1 https://github.com/noxrepo/pox.git ~/pox_tmp
cp controllers/pox_ai_controller.py ~/pox_tmp/ext/pox_ai_controller.py

After code changes: Copy controllers/pox_ai_controller.py into ~/pox_tmp/ext/ again before the next POX run so POX uses the latest controller code.

## Terminal 1 - start POX

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
PYTHONPATH=".:$HOME/pox_tmp" python3 "$HOME/pox_tmp/pox.py" pox_ai_controller

## Terminal 2 - run topology and traffic

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
sudo env PYTHONPATH="$PWD" python3 -m topologies.run_mininet_experiment \
  --controller-ip 127.0.0.1 \
  --controller-port 6633 \
  --profile mixed \
  --duration 30 \
  --output results/mininet_pox_run.json

Success signal: Mininet should write results/mininet_pox_run.json. POX writes decisions to results/pox_decisions.jsonl.

# 8. Full Mininet Run: Raw Controller

## Terminal 1 - start the raw OpenFlow controller

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
python3 -m controllers.raw_openflow_controller \
  --host 127.0.0.1 \
  --port 6633 \
  --metrics results/raw_decisions.jsonl

## Terminal 2 - run topology and traffic

cd "/mnt/c/Users/ROHAN A S GOWDA/OneDrive/Dokumen/computer_network_rohan"
sudo env PYTHONPATH="$PWD" python3 -m topologies.run_mininet_experiment \
  --controller-ip 127.0.0.1 \
  --controller-port 6633 \
  --profile mixed \
  --duration 30 \
  --output results/mininet_raw_run.json

# 9. Repeat Runs and Traffic Profiles

The experiment accepts five profiles. Replace mixed in the Mininet command with one of these values:

| Profile | Behavior under test |
| --- | --- |
| mixed | Runs VoIP, video, file, and web workloads together. |
| voip | Frequent small UDP packets; prioritizes low delay. |
| video | Steady large UDP packets; prioritizes bandwidth and stable delay. |
| file | Bulk TCP transfer; prioritizes maximum throughput. |
| web | Irregular TCP bursts; balances delay and congestion. |

# Example: a 60-second file-transfer test
--profile file --duration 60

Fair comparison: Use the same profile and duration for Ryu, POX, and Raw. Give each run a different output filename so results are not overwritten.

# 10. Stop and Clean the Network

Wait for the Mininet experiment to finish.

In Terminal 1, press Ctrl+C to stop the controller.

Clean any leftover Mininet namespaces, links, and switches.

sudo mn -c

Confirm that no controller is still using TCP port 6633 before the next run.

sudo ss -ltnp | grep 6633 || echo "Port 6633 is free"

# 11. Find and Read the Results

ls -lh results
cat results/mininet_ryu_run.json
cat results/mininet_pox_run.json
cat results/mininet_raw_run.json

| Metric | Meaning |
| --- | --- |
| pingall_loss_percent | Connectivity across all six hosts. A healthy run should normally be 0%. |
| pingall_seconds | Time needed to establish and test host-to-host connectivity. |
| iperf_h1_h6 | Measured throughput between h1 and h6. |
| traffic_seconds | Elapsed time for the selected workload. |
| *.jsonl decision log | Per-flow classification, chosen path, rule timing, and reroute details. |

# 12. Troubleshooting

| Problem | Fix |
| --- | --- |
| ryu-manager: command not found | Use $HOME/.local/bin/ryu-manager, or export $HOME/.local/bin in PATH. |
| Address already in use on port 6633 | Stop the previous controller with Ctrl+C. Check with sudo ss -ltnp ¦ grep 6633. |
| Mininet hosts cannot ping | Confirm the controller is already running, then run sudo mn -c and retry. |
| Open vSwitch is not available | Run sudo service openvswitch-switch start and sudo ovs-vsctl show. |
| ModuleNotFoundError for project modules | cd to the project root and export PYTHONPATH=$PWD before launching the controller. |
| POX cannot find pox_ai_controller | Copy the file into ~/pox_tmp/ext and launch the component name pox_ai_controller. |
| Permission denied from Mininet | Run only the Mininet experiment and mn -c with sudo; run controllers as the normal WSL user. |
| Old or duplicate Mininet processes | Stop controllers, run sudo mn -c, and start a fresh pair of terminals. |
| OneDrive path is difficult | Keep the project path in double quotes. For faster Linux I/O, optionally copy the project into ~/computer_network_rohan. |

# 13. Final Success Checklist

WSL reports Ubuntu-22.04 as version 2.

Open vSwitch responds to sudo ovs-vsctl show.

python3 -m ai.train_model creates the .joblib model.

The synthetic comparison creates CSV, JSON, and PNG files.

Ryu, POX, and Raw each connect to six OpenFlow switches.

Each Mininet run writes its own JSON result file.

pingall_loss_percent is 0.0 in a healthy run.

sudo mn -c completes after testing.

Known-good benchmark: The verified WSL2 runs for this project reached 0% ping loss and approximately 19.0/18.9 Mbits/sec between h1 and h6 for all three controllers.